Environmental Geology Lab II - TAMIU Flooding Lab

All water on our planet resides in temporary storage areas called reservoirs.  The largest reservoir for water is the oceans, but considerable water is present in the atmosphere (as gaseous water vapor), on land (in lakes, rivers and glacial ice) and under the ground (as groundwater).  Reservoirs are considered temporary storage facilities because water is constantly being transferred between them.  The Hydrologic Cycle is an attempt by geologists to quantify the exchange of water between the various reservoirs.  In its simplest description, the Hydrologic Cycle represents a balance between atmospheric precipitation (as rain, sleet, snow... etc.) and all of the reservoirs that receive this precipitation.  A basic equation my be written that summarizes this description:

(1)


P = ET + Q + I

where: 


P =
the amount of atmospheric precipitation available


ET = 
loss of precipitation through evaporation (heat from the sun) combined with




transpiration; evaporation from the leaves of plants as they utilize the water as part of their




metabolic cycle (commonly referred to as evapotranspriation)


Q = 
discharge; a measure of the amount of surface runoff that flows through rivers and streams


I =

infiltration; water that penetrates the ground and may become groundwater

Figure 1. Shows the components of the hydrologic cycle.
QUESTIONS:

1. Do you think that the rate of infiltration (I) will be significant in south Texas?  If the infiltration term (I) is 



negligible then simplify equation 1.

2. Compare the magnitude of ET versus Q in this region.  In general, which term do you think will be larger?  



Hint: is it common for there to be running water in the streams present in this region?

Figure 2. Histogram plotting the average PET for each month of the year 2001 from the Texas A&M Agricultural Experiment Station at Uvalde, Texas, (http://uvalde.tamu.edu/).

NOTE: THIS FIGURE IS REPRESENTATIVE OF PET VALUES IN SOUTH TEXAS!


In hot, arid regions, such as our own, the rate of evapotranspiration is often very high.  In fact, the potential for evapotranspiration (or PET; a value calculated based on temperature, humidity, wind speeds and amount and type of vegetation present) may often exceed the amount of precipitation available.  When this occurs all rainfall either evaporates before hitting the ground or shortly thereafter and there is none left over to replenish local rivers and streams (ie. P = ET and Q = 0).  In this case, actual evapotranspiration (ET) is less then potential evapotranspiration (PET).  

However, if precipitation (P) is greater than potential evapotranspiration (PET) then water will be available for discharge (Q > 0).  In this case, actual evapotranspiration (ET) is equal to potential evapotranspiration (PET).  

NOTE:  THAT THE ABOVE IS LIKELY TO BE TRUE DURING A PRECIPITATION EVENT.

QUESTIONS:

3. Which months have the highest PET values?  The lowest?  Is temperature directly correlated with PET in south Texas?  Explain!

4. How much PET is there during the month of July in Uvalde in mm?  In inches?

5. Over a 1 km2 area 10 cm of rain fell during the month of July.  Predict how much discharge (Q) is likely to occu in this area.
6. If this same area received the same 10 cm of precipitation during November calculate discharge in cm distributed over the entire 1 km2 area.

Figure 3. Digitizing a Sample Drainage Basin
Drainage basins have an irregular shape (in red). Within a drainage basin many streams flow (in blue).  These streams ultimately flow downhill into a main (trunk) stream that eventually exits the drainage basin.  One can conveniently measure the discharge (Q) within the entire drainage basin by placing a discharge meter (green dot) in the trunk stream where the water leaves the drainage basin.  

Parameters like precipitation can be remotely sensed using weather radar data.  This data has a spatial resolution of 1 X 1 km (1 km2).  Therefore, one can quantify precipitation within a drainage basin by dividing the basin into a grid of one square kilometer blocks.  Such a simplified drainage basin is shown at the bottom of Figure 3.

QUESTIONS:

7. What is the total area of the basin depicted in Figure 3 (bottom figure)?

8. Consider the result from question 6.  Spread discharge over the total area of the basin.  Determine how many m3 is potentially available for discharge.



HINTS:  (a) Convert discharge (Q) value in cm to meters and (b) convert 1 km2 to ??? m2
9. Note that 1 m3 is equal to 1000 liters. Convert answer from question 8 from m3 to liters.

Figure 4. Storm Events and Calculating Discharge
If the rate of PET and the amount of precipitation is known for a given area then this information may be used to calculate how much rainfall is lost during a storm event and how much is available as potential discharge.  In this exercise we have chosen a small river drainage basin where the PET (same as ET) is known (using Figure 2) and precipitation data (Figure 4) has been monitored and recorded by weather radar instrumentation.  Three types of storm events are considered:


(1) Winter precipitation event:  
Widespread; low intensity; associated with stratiform (low-lying) clouds 






connected with frontal systems; common in winter


(2) Summer convective:
Spatially isolated and small (commonly < 10 km2) high intensity; associate d precipitation event with localized episodes of atmospheric instability; common in summer


(3) Tropical storm event:
Widespread; very high intensity; fairly rare; usually in summer and fall

Example 1: Winter Precipitation Event

Example 2: Summer Convective Precipitation Event


Note for example 2:  Consider only P and ET in blocks that have significant precipitation!

Example 3: Tropical Storm Event


QUESTIONS:

10. In which example is the highest percentage of precipitation lost to ET?  The example with the least ET lost?

11. Convert calculated discharges for examples 1 to 3 into cubic feet per second!

Figure 5. Probability of Discharge Recurrence

QUESTIONS:

12. Typically, streams overflow their banks on average of once every other year.  For the trunk stream of our drainage basin how much discharge (in cubic feet per second) is needed to achieve a bankful state.

13. What is the recurrence interval for the thunderstorm and tropical storm of examples 2 and 3?  Is flooding likely to occur associated with these precipitation events?

14. Approximately how many times more rain is needed for a storm with a recurrence interval of one hundred years compared with the tropical storm of example #3.

Precipitation Intensity (mm)�
Precipitation Intensity (m)�
Number of Blocks�
Area of Blocks (square km)�
Area of Blocks (square m)�
Precipitation (cubic meters)�
�
256�
�
�
�
�
�
�
128�
�
�
�
�
�
�
64�
�
�
�
�
�
�
32�
�
�
�
�
�
�
16�
�
�
�
�
�
�
8�
�
�
�
�
�
�
4�
�
�
�
�
�
�
2�
�
�
�
�
�
�
1�
�
�
�
�
�
�
0�
�
�
�
�
�
�
Total�
--------------------�
--------------------�
--------------------�
--------------------�
�
�
--------------------�
--------------------�
--------------------�
--------------------�
--------------------�
--------------------�
�
ET (mm)�
ET (m)�
Area of Basin (square km)�
Area of Basin (square m)�
--------------------�
ET (cubic meters)�
�
�
�
�
�
--------------------�
�
�
--------------------�
--------------------�
--------------------�
--------------------�
--------------------�
--------------------�
�
--------------------�
--------------------�
Q = P - ET�
Discharge (cubic meters per day)�
Seconds per day�
Discharge (cubic meters per second)�
�
--------------------�
--------------------�
--------------------�
�
�
�
�






Precipitation Intensity (mm)�
Precipitation Intensity (m)�
Number of Blocks�
Area of Blocks (square km)�
Area of Blocks (square m)�
Precipitation (cubic meters)�
�
256�
�
�
�
�
�
�
128�
�
�
�
�
�
�
64�
�
�
�
�
�
�
32�
�
�
�
�
�
�
16�
�
�
�
�
�
�
8�
�
�
�
�
�
�
4�
�
�
�
�
�
�
2�
�
�
�
�
�
�
1�
�
�
�
�
�
�
0�
�
�
�
�
�
�
Total�
--------------------�
--------------------�
--------------------�
--------------------�
�
�
--------------------�
--------------------�
--------------------�
--------------------�
--------------------�
--------------------�
�
ET (mm)�
ET (m)�
Selected Area (square km)�
Selected Area (square m)�
--------------------�
ET (cubic meters)�
�
�
�
�
�
--------------------�
�
�
--------------------�
--------------------�
--------------------�
--------------------�
--------------------�
--------------------�
�
--------------------�
--------------------�
Q = P - ET�
Discharge (cubic meters per day)�
Seconds per day�
Discharge (cubic meters per second)�
�
--------------------�
--------------------�
--------------------�
�
�
�
�






Precipitation Intensity (mm)�
Precipitation Intensity (m)�
Number of Blocks�
Area of Blocks (square km)�
Area of Blocks (square m)�
Precipitation (cubic meters)�
�
256�
�
�
�
�
�
�
128�
�
�
�
�
�
�
64�
�
�
�
�
�
�
32�
�
�
�
�
�
�
16�
�
�
�
�
�
�
8�
�
�
�
�
�
�
4�
�
�
�
�
�
�
2�
�
�
�
�
�
�
1�
�
�
�
�
�
�
0�
�
�
�
�
�
�
Total�
--------------------�
--------------------�
--------------------�
--------------------�
�
�
--------------------�
--------------------�
--------------------�
--------------------�
--------------------�
--------------------�
�
ET (mm)�
ET (m)�
Area of Basin (square km)�
Area of Basin (square m)�
--------------------�
ET (cubic meters)�
�
�
�
�
�
--------------------�
�
�
--------------------�
--------------------�
--------------------�
--------------------�
--------------------�
--------------------�
�
--------------------�
--------------------�
Q = P - ET�
Discharge (cubic meters per day)�
Seconds per day�
Discharge (cubic meters per second)�
�
--------------------�
--------------------�
--------------------�
�
�
�
�









